Master’s Degree in Computer Science

Academic year 2024/2025

INTERNET OF THINGS

AND SMART CITIES

Prof. Marco Giordani

Written by Michael Amista’

A course offered by the School of Engineering (DEI) –

Master’s Degree in ICT for Internet and Multimedia

Index of contents

Internet of Things and Smart Cities course introduction	3

1. Internet of Things	4

1.1 Introduction	4

1.2 System architecture	13

1.2.1 Structure and definitions	13

1.2.2 Data plane functions	19

1.2.3 Control plane functions	25

1.2.4 QoS and performance	26

1.2.5 Energy constraints	26

2. Smart Cities	26

# Internet of Things and Smart Cities course introduction

Course prefix to use as email object: IOTSC

From a customer perspective the IoT is the ability to use sensors and other devices to connect things in a way they work together.  IoT finds its main use in smart home, smart cities and industrial systems.

Obviously, the main issue when we talk about IoT is the reliability of the collected data and this is a big challenge in protecting our privacy, we will deal with that during the course. Another important problem is the energy consumption: many sensors and devices have a limited amount of battery and so battery optimization considerations are needed. Interoperability is another problem: there are many manufacturers and many different devices, so it is important to have a common standard to make different devices work together. Furthermore, depending on the number of devices, larger networks are created, and it is crucial that these networks are well handled in a way to be able to manage the large number of nodes.

The course is more focused on IoT technology and, only at the end, on its application to build one IoT scenario: smart cities.

Course schedule:

- Frontal lectures.

- LAB experiences: it starts in November and there will be 3(+1) practical lab experiences. We will use Arduino as an emulated sensor to send data to IoT LoRa gateway and we will see how to process those data to return an output.

- Guest lectures (even those part of the written exam).

Slides (and attendance) are enough to pass the exam.

The exam consists of two parts:

- PART 1 (up to 28/33 points): Written test (mandatory) that consists of both multiple-choice questions and open questions (even the guests lectures can be part of the written exam) with a duration of 1 hour and half.

- PART 2 (up to 5 extra points): LAB assessment (optional) which is “only” a replication of what done during lab lectures so if you follow the lab you should not have to study anything else to work on the assessment. The assessment can be submitted in December, before the written exam session. To submit the assessment, you must follow all lab lectures.

# 1. Internet of Things

## 1.1 Introduction

The term “IoT” is characterized by two fundamental concepts: internet and things.

The precursors of the modern Internet (ARPANET, CSNET, NSFNET) were resource sharing networks: computers were bulky and expensive, and researchers used nationwide connections to access them from far away. As personal computers became ubiquitous, and packet-switched traffic was ported to the ubiquitous telephone network, the Internet became the means for people all over the world to communicate with each other. In 1990, Tim Berners-Lee defines HTTP and HTML, leading to the explosion of the World Wide Web.

From interconnecting computers and people now IoT interconnects personal devices. The goal is to create a network, an architecture able to sustain all the different interconnected devices that shape the network. The basic idea is the interconnection needs to be invisible to users who do not have to do anything, everything is automatically, data are processed, and the users have an automatic response having so an autonomous network that works without human interaction.

WPAN is a connection technology specifically developed for IoT sensors. IoT sensors can use many technologies to communicate each other, from the IoT nonspecific ones (e.g., WLAN) to more specific ones (e.g., WPAN).

We will see how IoT can decrease the costs and optimize operations, that’s why the IoT revenues are so high. Some statistics: the number of Italian enterprises using IoT technologies is around 32%.

But what is IoT? We need a definition. There are a lot of definitions of what IoT is, all of them are correct but we will use a more formal definition than the ones that appear in the textbooks.

“Internet of things means connecting every thing.”

The key part of the definition of “Internet of Things” is the Internet, where Internet means IP protocol: No IP  No IoT. So, the definition we will use during the course will be the following:

“Internet of Things is a paradigm according to which every thing, real or

virtual, is assigned an IP(v6) address and can be reached (for example for

sensing or actuating purposes) via the standard Internet Protocol stack.”

Basic concept: if you assign an IP address to a device, that device is part of an IoT network.

An example of a “Smart” IoT system

- One typical evening planning next working day...

- Tomorrow first office meeting at 8:30 am.

- Typical car trip in these days: 1 hour time.

- 45 minutes to wake up and get ready.

- I decide to set my alarm to wake up at 6:45 am.

What could (will) possibly go wrong?

- At 4:30 am it starts snowing

- Truck obstruction along the usual path

- Traffic congestion on alternative paths

- No parking at destination

- Bathroom cold when having shower

- Coffee cold when having breakfast

- Left my car keys at home when in garage

- Elevator busy when leaving my flat

Leaving 10 min. late + 30 min. additional travel time  missed the meeting!

An IoT approach to improve this scenario…

We want to have an integrated system made by smart things that make the life easier for the final consumer and so to take actions without human intervention.

IoT high-level architecture

The core parts of an IoT system are sensors that produce data to be consumed. Data are collected by a gateway and then they are processed (trigger some actions) or only visualized (only to monitor the system making sure everything goes well). The actuator is another type of device that implements an action based on the result of data processing.

Data collection

- Sensors acting as a physical-cyber interface that monitors and reports states of some physical entity or device.

- Produce a digital representation suitable for use in the cyberspace.

- Relatively early in the process, metadata needs to be captured and used to annotate the data. In IoT systems, metadata generally describe the nature and context of data capture, such as the sensor type, its location, and in some cases structural relationships to other elements of the system (a sort of appendix to real data, used to store other information).

Processing and visualization:

- Types of IoT data processing:

  - (Simple) control loop algorithms performed on the incoming data as they arrive.

  - Sophisticated forms of analytics and machine-learning algorithms based on past behaviours and observations of the system.

  - Common data processing steps:

    - Sampling, aliasing, quantization, saturation, hysteresis and non-linearities, calibration, error propagation, optimization and predictions (we’ll see…).

- In industrial and complex control systems, it is customary to visualize the system state and points of interest to system operators  digital twin / dashboard.

  - System state, notifications, alarms when faults or anomalous behaviours are detected.

Acting:

- Acting upon insights and predictions is the output and the ultimate purpose of deploying IoT systems.

- Common types of actions:

  - From simple remote actuation initiated by operators in response to visualized conditions in a basic monitoring configuration to automated guidance of control points.

- Actions can be implemented as direct actuation or indirectly, in the form of advice to system operators or optimizations resulting in adjustments to the manufacturing process.

- Identification of cause of failures and anomalous conditions followed by direct or indirect execution of the appropriate remediation actions.

A more technical IoT architecture

Let’s explore the different elements that shape this more technical architecture.

IoT applications

These are the actual use cases and services that benefit from IoT technology. We need to understand them to design appropriate networks. They span a wide range of industries, including smart cities, health monitoring, industrial automation, transportation, and more. These applications rely on IoT sensors and gateways to collect and process data to offer services like predictive maintenance, asset tracking, or smart energy management.

Examples of IoT applications (look at slides for the whole list): a common example is the IoT usage in Industry 4.0 where sensors are used to collect data to optimize industrial operations. Another IoT application is related to smart shops as self-checkout or retail store optimization (sensors are used to study the shop areas on which the customer is more focused to understand how shop items can be placed to attract customers  look at the example here below where the visors are used to collect data and statistics on customer behaviour).

Another IoT use case is related to healthcare as remote patient monitoring or medical alert systems (sensors are used to detect possible issues). IoT touches other scenarios such as smart cities/homes, transportation and automotive. Autonomous driving is a quiet different use case respect to the others, here the sensors are more complex and due to this is a more challenging use case on which work.

IoT sensors

- Sensors can measure some quantity in the environment (e.g., a thermometer). In simple terms they sense/perceive the environment.

- Actuators can do something in the environment (e.g., turn on the heating system). Actuators act in response to some measures.

- Sensors can be simple or more complex. In case of the first they delegate the major computation to other external nodes (inside the edge layer - gateways - or even outside) because they are not able to do the operation. In case of the second there are scenarios where the sensors can implement some local operations to act immediately because delegating the action to others will take too much time. Basically, most of the sensors are of the first type.

- Some sensors and things are designed to connect directly to the Internet and communicate with applications and services residing in the cloud.

  - It may require external computation platforms given energy constraints.

  - Security cameras, fire sensors, thermostats, appliances, and power meters, …

IoT gateways

Sensors connect to the Internet (Cloud) using intermediaries, such as gateways. Gateways (and fog nodes) are usually more powerful devices  connection via local network links, often wireless, such as: ZigBee, variants of 802.5.14 networks, Bluetooth, and low-power Wi-Fi, LoRa, NB-IoT.

Gateways usually provide wide-area connectivity and edge processing for the attached sensors that may come in the form of protocol conversion, data storage and filtering, event processing, and analytics. Gateway is much closer to the end user respect to the Cloud (where processing phase typically happens) and sometimes, as said before, can be useful to process the data in a little amount of time and that’s why gateways implement edge processing.

Gateways can also store data; this is more convenient instead of saving data on sensors. This avoids data fragmentation problem since a gateway connects more sensors and it has a full knowledge from more sources (data from different sources stored in a single node). Storing data on gateways is also convenient in terms of complexity  remember that sensors are simple devices.

Communication layers

Enables a vast array of edge devices and things to exchange messages with each other, the rest of the IoT system, and ultimately the Internet.

Communication layers may include a variety of wireless and wired links, spanning local areas and including long-haul connections. Represent a complex infrastructure of links, bridges, and routers that can transport payloads from local point-to-point segments all the way to any endpoint and application on the Internet.

The Cloud

“Back-end” processing is depicted by a generic cloud. Cloud represents a collection of servers and other data storage systems that can be accessed via internet. Sensors are connected to gateways and gateways provide the end-to-end connection with the cloud.

- Data from a variety of diverse sources are aggregated and processed for optimization and discovery of global trends and relations.

- Depending on nature and real-time requirements, sensor data may be processed “inflight” as streams (a simple check on a value  constantly monitor the condition of a system), stored for post-processing and archival purposes, or both.

- May also contain some common services such as large-scale storage, analytics processing engines, data visualization and graphing, as well as management functions such as security and provisioning.

- Machine learning (ML) and artificial intelligence (AI) algorithms are usually operated in the cloud where they can work with large aggregations of data.

Control plane

- Data plane / user plane: main IoT functionalities: collect, process, and act on data.

- Control plane: task of keeping the IoT infrastructure itself running and secure.

  - Service configuration and aggregation

  - Service problem management

  - Service quality management

  - Resource provisioning

  - Trouble and anomaly management

  - Performance management

IoT: why today? A confluence of technological and infrastructure developments centered around the Internet forms much of the basis and impetus for the construction of IoT systems.

- Industry 4.0 and digitalization.

- Sensors – installed base, variety, lower cost, and easier integration.

- Multi-sensors miniaturization and mass production.

- Smart phones – sensors, gateways, and UI devices.

- Cloud computing – global and capacity on demand.

- AI and ML technologies – actionable insights with IoT data.

- Internet – technology, global infrastructure, and users.

- Pervasive communications: mobile devices always connected to the Internet.

MAIN PROBLEMS IN IoT

1) The energy problem

- Sensors might be placed in hard-to-reach locations: they need to be battery-powered and work for months or years.

- Sometimes they have solar panels or other energy harvesting methods, but we need to be careful: size and cost matter!

- Energy has a significant impact on every IoT operation: communication, computation, sensing, actuators...

- Due to the energy problem, sensors are often optimized to consume little power:

  - We cannot operate on an always-on basis  duty cycles (for instance, having a duty cycle of 1% means that sensor can transmit only for 1% of the time, the rest of the time the sensor does nothing).

One can also consider a sleeping-mode where sensors are active for a certain amount of time and then they just sleep  this solution is not the best since during the sleeping time sensors cannot detect anything and also the waking-up process can consume a lot of battery if the network is not properly designed.

  - We need to consider every aspect: any calculation or long transmission is expensive.

- For this reason, sensors often transmit very few bytes at relatively long intervals (no transmissions for hours or days in some applications).

- Limited processing capabilities (and so latency concerns).

2) The security problem

- The simplicity of IoT devices also makes them hard to secure.

- Impossible to run strong cryptography.

- Difficult to patch and update to solve vulnerabilities.

- Connected by design to the open Internet.

- There are possible security solutions against hackers, but there is also a privacy issue: what do sensors say about our lives? Who gets to see the data?

- Sensor data can tell a lot about our lives:

- Location data can identify movements and activities.

- Domotic systems tell stories about our habits.

- Data are often used by IoT companies and law enforcement agencies.

3) The scalability problem

- Cellular networks and wireless technologies were designed for few devices with high requirements (e.g., humans streaming videos).

- In the IoT, we have thousands or millions of devices with very little data to send.

  - New access mechanisms.

  - Sensor identification problems.

  - How do we get the data with the minimum number of transmissions?

4) The reliability problem

- The “converse” problem (what do we need to ensure that we get the relevant information) can be crucial in industrial and safety systems:

  - How do we identify anomalous (and potentially hazardous) situations?

  - Anomalies can be single-sensor or multi-sensor.

  - How do we trust that there is an anomaly when sensors are unreliable?

## 1.2 System architecture

### 1.2.1 Structure and definitions

Now we will focus on the edge layer of the IoT architecture, so on sensors and gateways.

- Client/server: the information sources explicitly know the destination about the destination of their messages. Not convenient in IoT networks as the publish/subscribe paradigm.

- Publish/subscribe: the sources send (publish) their messages on a logical distribution channel, to which zero, one or more interested destinations may subscribe, implementing a one-to-many communication pattern. In the client/server we need to establish one-to-one communication. In the publish/subscribe the nodes that publish data do not publish data for a specific subscriber, so there is a one-to-many communication (many nodes can subscribe to the same list if they are interested in that data). This approach gives more freedom than the client/server one.

- Looking at the information carried by the network, we can identify 3 flows:

  - Streams of asynchronous events (generated autonomously).

  - Measurements transmitter periodically or on-demand.

  - Commands coming from the processing platform. Subscribers may require some data to the publisher which is triggered by the subscriber request, and so the required data needs to be sent.

Network architectures

Three network architectures:

- One-level architecture: the endpoints know about the addressing scheme. The end-node needs to implement the communication protocol and so it becomes a network device.

  - Direct IoT connectivity to the Internet protocol stack.

  - Typically implemented with cellular network technology.

- Two-level architecture: the last segment is unaware of addressing aspects (endpoints send messages which are not geographically routable) and a gateway handles the communication protocol.

  - End nodes are generally resource and/or energy constrained (e.g., sensors).

  - Typically implemented with specific IoT technologies: Low Power Wide Area (LPWA) technologies.

  - Example: LoRa, SigFox, NB-IoT, etc.

- Two-level architecture: the endpoints are not resource-constrained and implement the TCP/IP protocol stack and due to that the gateway is no more useful. The traffic is managed by an intermediate router.

  - Typically implemented with Ethernet LANs or Wi-Fi, etc.

Wide area networks (WANs)

- WANs are (generally wired) TCP/IP networks.

- The WAN must be shared among many users and applications (economic reasons)  potential security and access issues. There are mechanisms to segregate and reserve the traffic:

  - Virtual Private Network (VPN): network that takes resources (bandwidth capacity, switching and routing functions, firewalls and other security appliances etc.) and reserves them to a defined group of users/endpoints. Users external to this group cannot communicate directly with the users of the group. This system increases the complexity since we need to manage different VPNs.

  - Users outside of a group cannot communicate directly with users of the group.

  - Use private IP addressing and private routing protocol instances.

In the above example the WAN is shared by two systems. The system1 collects data from 3 sides. The system2 records videos via cameras. Since both systems share the same network, two VPNs were created and the virtual routers (VR) of both networks provide access to the “real” network. In fact, both systems operate on two separated VPNs avoid security and collision problems.

TOPOLOGIES

1) BUS

- Every node taps into a common medium (the bus), signals may collide with each other.

- Need to arbitrate who will get the bus.

- The common medium is the bottleneck (to be shared)  single point of failure (if the bus is not available for some reason all the nodes are cut off).

- Use CSMA/CD

- Example: (old, 10BASE2, 10BASE5) Ethernet.

2) (TOKEN) RING

- Nodes are in a ring: receive from predecessor and send to successor.

- Need to arbitrate which node can access the ring (repeaters at every link).

- The common medium is the bottleneck (to be shared)  single point of failure.

- Token-ring topology:

  - A node wishing to transmit waits for the receipt of a token, removes it from the ring, and places its message.

  - Then, the node holding the token passes it along.

  - Eliminates collisions and can increase throughput.

  - Need a mechanism to preserve token integrity and to regenerate it if necessary (e.g., when nodes are powered off).

  - The communication cycle can be very long and there is no coordinator that manages the whole system.

3) STAR

- The master is at the center, other nodes are slaves linked to the master.

- This approach is better than the ring topology since here we have the coordinator which is the master.

- Slaves communicate via master.

  - Easy to arbitrate among slaves (master decides).

  - Not scalable (the master is the bottleneck).

  - Good for small networks or that requires predictable performance.

  - Master failure shutdowns  single point of failure.

4) (PARTIAL) MESH

- IoT network use this approach. Every node can communicate with others, deleting so the client/server one-to-one communication approach.

- Nodes are arranged according to connectivity graph.

  - Each node has a dedicated point-to-point connection to every other node.

  - Number of physical links/connections is n(n-1)/2 (if full-duplex). The problem is the scalability: if we have many nodes the number of links that we have to establish is really huge and expensive.

  - Reduces traffic forwarding issues (+ no shared medium collisions).

  - Ensures robust and private/secure network.

- Partial mesh: due to the scalability problem this solution represents a better approach. A subset of links representing direct connectivity between a subset of nodes  some nodes are used as intermediaries.

ENDPOINTS

A sensor is a device that detects some measurable aspect of the physical world state (stimulus) and converts it to a processable output, e.g., an electrical signal. There are thousands of sensor types, that can measure almost anything:

- Pressure, humidity, air quality

- Fingerprints, facial features, iris patterns

- Microphones, touch-sensitive surfaces, radars

- Light, smoke, chemical sensors

- Temperature

- Force

- Positioning

- …

Types of sensors:

- Fixed: stable position within the network topology (wireless vs. wired)  so basically sensors that are always in the same physical position.

  - No need to re-authenticate, stable address and routing, simple management, etc.

  - Can be mains powered.

  - Can have permanent bad wireless connectivity conditions, if bad deployment.

- Mobile: must be connected to a cellular or LPWA network.

  - May experience a degraded service only occasionally.

  - Limited mobility constraints (e.g., handover).

  - Cannot be mains powered.

- Nomadic: can change their physical position but stay in the same place for the whole duration of a communication session.

  - Must re-authenticate, but do not require tracking functions.

- Active: need to run a current or send a signal to get a measurement (e.g., clocks, strain gauges, radar).

- Passive: use the physical phenomenon itself (e.g., piezoaccelerometers, GPS, thermocouples). This does not mean that passive sensors consume zero energy!

Actuators are an output part of the IoT system that performs direct actions. Actuation provides the means to implement control actions as and when determined by the algorithms or system operators.

- Digital actuators: On-off action. Past the appropriate signal conditioning can trigger relays or change the state of a thing, such as a power switch which can result in turning a light on or off.

- Analog actuators: Produce continuous signals that can be used to drive devices. For example, controlling the speed of a motor or producing sound in headphones.

Gateways

- It links sensors and things at the edge with higher levels of system processing hierarchy and the cloud.

- It performs or assists data collection.

- It provides Internet connectivity to transfer of sensor data to other components.

- Security boundary between things with varying levels of security.

- Optional: data storage, event and alert processing, and control (automation).

- Advanced: analytics.

Fog nodes

- Etimology: bring the cloud to the ground.

- Continuum between the edge and the cloud by placing additional computation and storage capability closer to the edge.

- Perform local processing (e.g., data reduction, filtering, and front-end analytics) with potentially lower latency than in the cloud.

- Fog nodes tend to be more powerful but are otherwise not architecturally or functionally different from the edge gateways.

### 1.2.2 Data plane functions

Data acquisition and processing

In order to get a digital signal from a sensor, we need to account for:

1) Sampling

First, we sample: we take (regular) measurements over time, and represent the signal with those measurements (Nyquist theorem: if the sampling is twice as fast as the signal, sampling has no errors).

Example: If a signal is thought to have a maximum frequency between 1000 Hz and 4000 Hz, which of the following would be the most appropriate sample rate?

2) Aliasing

If we do not sample “enough,” we have aliasing: patterns in the sampled version. A high frequency component in the frequency spectrum of a signal takes the identity of a lower frequency component in the same spectrum of the sampled signal. This falsely estimated signal will be indistinguishable from (i.e., be an "alias" to) another signal having that true lower frequency.

There are tools to avoid this, but we need to know it is an issue. Example: 15 KHz vs. 6 KHz sampling.

3) Quantization

Digital values are finite: we cannot represent continuous values precisely! Quantization always introduces an error, which is the sensor’s natural error.

4) Saturation

Sensors often have an upper limit, which is either physical or dictated by the maximum quantization level: in this case, we have saturation (all higher values are mapped to the same output). This is what happens when you take a picture with high exposure: all lighter points

are bright white!

5) Data acquisition and processing: Hysteresis

Sometimes, a sensor (e.g., a thermometer) takes a while to track the process because the physical nature of the measurement is slow  Hysteresis error when the delay of sensor measurements depends on the magnitude of the change.

Example: When you apply pressure to a sensor and then release that pressure, the sensor’s output should in theory return you to the same value it started at before that specific pressure cycle. However, in reality there are often small differences between the initial value and the final value after a pressure cycle.

- From 0 to 100 psi

- 0 psi  0 V

- 100 psi  2500 V

- 0 psi  0.05 V

  - 0.02% / 1 psi hysteresis

5) Data acquisition and processing: Non-linearities

Ideally, sensors should also have linear responses: in reality, there are often nonlinear effects that we need to adjust for.

Example: A linear sensor’s output will change by equal amounts for equal pressure changes across its entire range.

- From 0 to 10 bar

- 0 V at 0 bar pressure

- 5 V at 10 bar pressure

- But at 3 bar pressure, it outputs 4.2 V instead of 3.75 V.

  - ≤2% non-linearity of full scale

6) Calibration

- Sensors often require calibration to give proper results:

  - Bias is a constant error term in one direction.

  - Distortion is due to non-linearity.

- By measuring a series of known quantities, we can compensate for these issues (however, external factors such as temperature can throw off calibration!)

- Example: Using the data from the calibration sheet, we see that the deviations are all less than the maximum deviation allowed of 0.5%.

7) Error propagation

- Calculations can change the error of a measurement, as the errors are also considered in the function you compute.

  - Non-linear functions can have weird effects with measurement errors.

  - Derivation (e.g., computing velocity from position measurements) is brittle: since the measurements are close together, noise can have a huge effect.

- Integration/averaging are robust (rely on many points, and noise is compensated).

- Example: Calculating heat index

  - Temperature sensor: measured (30°) with uncertainty ±0.5

  - Humidity sensor: measured (70%) with uncertainty ±2%

  - Heat index = T + 0.33×RH−0.7 = 52.4

    - Uncertainty with error propagation: 0.828

External data communication

- Sensors data may need to be shared with external parties, such as other peer nodes at the edge, fog, and cloud levels. Messages are exchanged between IoT nodes may be delivered using push or pull modes.

  - Pull: a message is explicitly requested from the source (server) by its recipient, an IoT client.

  - Push: the data source and the gateway send data when the specified conditions are met, such as arrival of the data sampling time mark, or exceeding of the measurement threshold.

- Publish/subscribe system: data sources “publish” data to a known place, and authorized clients can “subscribe” to data of interest and receive related messages. Publishers do not need to know the identity or number of their receivers (subscribers), and the receivers do not need to know the identity or state of their publishers.

External data communication: how to share sensor’s data?

- Some processes (e.g., an electricity meter, a room thermostat) do not need to be sampled very often.

  - If we only care about cumulative measures (how much electricity was consumed in total), we do not need frequent samples!

  - On the other hand, faster processes need frequent samples, as process can change faster.

- Why don’t we transmit all the time?

  - There are several reasons why we cannot transmit all the time, but we will concentrate on two of them in this first part of the course: Limited bandwidth. Limited energy.

  - Each transmission (and, in fact, each activity the sensor performs) costs energy, as well as blocking the wireless channel for other transmissions.

External data communication: some solutions to send sensors’ data more efficiently

1) Interpolation

Interpolation is the mathematical tool to infer missing data from sparse samples. We have to make some assumptions on the nature of the process to be able to interpolate: if we have the correct model, we can send a lot fewer data samples!

Interpolation can also be dangerous: we can underfit (select a model that is too simple) or overfit (select a model that is too complex and ends up following random noise).

2) Spatial correlation

- Data are usually correlated in space:

  - Traffic on either side of a bridge.

  - Temperatures close together.

  - There can be easy natural relations (points close together are similar) or structural relations (connected points are similar).

- Since measurements have errors, correlations help us improving the quality of estimates: we can make statistical calculations and estimates to figure out what is happening in points (like the purple one on the bridge) where we have no sensors, or refine our estimates.

3) Temporal correlation

- Data are also correlated in time:

  - Traffic in one spot

  - Temperature measured by one sensor

- The relation here can be more or less complex, and depend on space as well (e.g., line at a traffic light), but we can also exploit this to get better data with fewer transmissions.

- Correlations can be misleading and dangerous! When relying on statistical information to reconstruct missing data or make decisions, we need to be aware that random chance is always a possibility.

Local control

- Provided in systems where latency is critical and/or edge autonomy is required.

- Local control enables the edge node to execute pre-defined control sequences when a specified condition occurs on some combination of local data.

  - If This Then That: scenes where lights and heating/cooling are adjusted accordingly when users are leaving or approaching their homes.

- Node-RED: provides a graphical user interface to construct data flows with the functional processing modules and actuation output stages.

- In the extreme case of temperature, the script triggers the device shutdown.

Data storage

- Data storage for the acquired data may be provided by the gateway.

- The existence of local storage provides the flexibility to conserve network bandwidth and to reduce the load on cloud resources by selectively reporting data that meet certain conditions.

  - Depending on the nature of the physical thing being measured, only some of the data points may be of interest. E.g., successive minor changes may be of little interest, but those that exceed a comfort guard band or change by a significant percentage or amount from prior readings are.

  - Qualifying readings may be defined as events and reported only when they occur, with raw readings stored in the local database should they be required for auditing or subsequent analysis.

Edge analytics

- Edge analytics for the data may be provided by the gateway or the fog node.

  - Proximity to data sources: low latency

  - Ability to process high-frequency data samples (at the rate of the sensor)

  - Conservation of network bandwidth by not sending data to the cloud.

  - Can operate even in disconnected mode.

  - Analytics generally require AI/ML models:

    - Insufficient computational resources at the edge.

    - No data aggregation of edge/local data.

- IoT systems should be implemented using design tools and practices that facilitate flexible allocation of functions (in tandem between edge and cloud).

Analytics placement

- Optimal placement of data and processing functions is an age-old tradeoff in distributed systems.

- It is based on whether it is more cost-effective to move the data to the computation or to move the computation to the data, depending on:

  - Availability and cost of bandwidth

  - Latency requirements for time-critical operations

  - Local autonomy of operations, including disconnected mode

  - Security and data control or privacy concerns

  - Cost and complexity of managing distributed nodes with computing and storage

  - Available energy resources

  - Available computational capacity

  - …

### 1.2.3 Control plane functions

Overview

- Fully functional gateways may be fairly sophisticated computational and communications nodes that host and execute a variety of services.

- They need to be properly secured and managed.

- Main control functionalities:

  - Fault management (troubleshooting, error logging, and recovery).

  - Remote monitoring, control, administration, and diagnostics.

  - Remote firmware and software updates.

  - Security updates.

  - Metering (network bandwidth and software usage) and supervision.

  - Provisioning and authentication.

Slide 46/83

### 1.2.4 QoS and performance

…

### 1.2.5 Energy constraints

…

# 2. Smart Cities

…