Master’s Degree in Computer Science

Academic year 2024/2025

INTERNET OF THINGS

AND SMART CITIES

Prof. Marco Giordani

Written by Michael Amista’

A course offered by the School of Engineering (DEI) –

Master’s Degree in ICT for Internet and Multimedia

Index of contents

Internet of Things and Smart Cities course introduction	3

1. Internet of Things	4

1.1 Introduction	4

1.2 System architecture	13

1.2.1 Structure and definitions	13

1.2.2 Data plane functions	17

2. Smart Cities	18

# Internet of Things and Smart Cities course introduction

Course prefix to use as email object: IOTSC

From a customer perspective the IoT is the ability to use sensors and other devices to connect things in a way they work together.  IoT finds its main use in smart home, smart cities and industrial systems.

Obviously, the main issue when we talk about IoT is the reliability of the collected data and this is a big challenge in protecting our privacy, we will deal with that during the course. Another important problem is the energy consumption: many sensors and devices have a limited amount of battery. Interoperability is another problem: there are many manufacturers and many different devices, so it is important to have a common standard to make different devices work together. Furthermore, depending on the number of devices, larger networks are created, and it is crucial that these networks are well handled.

The course is more focused on IoT technology and, only at the end, on its application to build smart cities.

Course schedule:

- Frontal lectures.

- LAB experiences: it starts in November and there will be 3(+1) practical lab experiences. We will use Arduino as an emulated sensor to send data to IoT LoRa gateway and we will see how to process those data to return an output.

- Guest lectures (even those part of the written exam).

Slides (and attendance) are enough to pass the exam.

The exam consists of two parts:

- PART 1 (up to 28/33 points): Written test (mandatory) that consists of both multiple-choice questions and open questions (even the guests lectures can be part of the written exam) with a duration of 1 hour and half.

- PART 2 (up to 5 extra points): LAB assessment (optional) which is “only” a replication of what done during lab lectures so if you follow the lab you should not have to study anything else to work on the assessment. The assessment can be submitted in December, before the written exam session. To submit the assessment, you must follow all lab lectures.

# 1. Internet of Things

## 1.1 Introduction

IoT is characterized by two fundamental concepts: internet and things.

The precursors of the modern Internet (ARPANET, CSNET, NSFNET) were resource sharing networks: computers were bulky and expensive, and researchers used nationwide connections to access them from far away. As personal computers became ubiquitous, and packet-switched traffic was ported to the ubiquitous telephone network, the Internet became the means for people all over the world to communicate with each other. In 1990, Tim Berners-Lee defines HTTP and HTML, leading to the explosion of the World Wide Web.

From interconnecting computers and people now IoT interconnects personal devices. The goal is to create a network, an architecture able to sustain all the different interconnected devices that shape the network. The basic idea is that interconnection needs to be invisible to users who do not have to do anything, everything is automatically, data are processed, and the users have an automatic response having so an autonomous network that works without human interaction.

WPAN is a technology specifically developed for IoT sensors. IoT sensors can use many technologies to communicate each other, from the nonspecific ones (e.g., WLAN) to more specific ones (e.g., WPAN).

We will see how IoT can decrease the costs and optimize operations, that’s why the IoT revenues are so high. Some statistics: the number of Italian enterprises using IoT technologies is around 32%.

But what is IoT? We need a definition. There are a lot of definitions of what IoT is, all of them are correct but we will use a more formal definition than the ones that appear in the textbooks.

Internet of things means connecting every thing.

The key part of the definition of Internet of Things is the Internet, where Internet means IP protocol: No IP  No IoT. So, the definition we will use during the course will be the following:

“Internet of Things is a paradigm according to which every thing, real or

virtual, is assigned an IP(v6) address and can be reached (for example for

sensing or actuating purposes) via the standard Internet Protocol stack.”

If you assign an IP address to a device, that device is actually part of an IoT network.

An example of a “Smart” IoT system

- One typical evening planning next working day...

- Tomorrow first office meeting at 8:30 am.

- Typical car trip in these days: 1 hour time.

- 45 minutes to wake up and get ready.

- I decide to set my alarm to wake up at 6:45 am.

What could (will) possibly go wrong?

- At 4:30 am it starts snowing

- Truck obstruction along the usual path

- Traffic congestion on alternative paths

- No parking at destination

- Bathroom cold when having shower

- Coffee cold when having breakfast

- Left my car keys at home when in garage

- Elevator busy when leaving my flat

Leaving 10 min. late + 30 min. additional travel time  missed the meeting!

An IoT approach to improve this scenario…

We want to have an integrated system made by smart things that make the life easier for the final consumer and so to take actions without human intervention.

IoT high-level architecture

The core parts of an IoT system are sensors that produce data to be consumed. Data are collected by a gateway and then they are processed (trigger actions) or only visualized (only to monitor the system making sure everything goes well). The actuator is another type of device that implements an action based on the result of data processing.

Data collection

- Sensors acting as a physical-cyber interface that monitors and reports states of some physical entity or device.

- Produce a digital representation suitable for use in the cyberspace.

- Relatively early in the process, metadata (a sort of appendix to real data, used to store other information such as the location) needs to be captured and used to annotate the data. In IoT systems, metadata generally describe the nature and context of data capture, such as the sensor type, its location, and in some cases structural relationships to other elements of the system.

Processing and visualization:

- Types of IoT data processing:

  - (Simple) control loop algorithms performed on the incoming data as they arrive.

  - Sophisticated forms of analytics and machine-learning algorithms based on past behaviours and observations of the system.

  - Common data processing steps:

    - Sampling, aliasing, quantization, saturation, hysteresis and non-linearities, calibration, error propagation, optimization and predictions (we’ll see…).

- In industrial and complex control systems, it is customary to visualize the system state and points of interest to system operators à digital twin / dashboard.

  - System state, notifications, alarms when faults or anomalous behaviors are detected.

Acting:

- Acting upon insights and predictions is the output and the ultimate purpose of deploying IoT systems.

- Common types of actions:

  - From simple remote actuation initiated by operators in response to visualized conditions in a basic monitoring configuration to automated guidance of control points.

- Actions can be implemented as direct actuation or indirectly, in the form of advice to system operators or optimizations resulting in adjustments to the manufacturing process.

- Identification of cause of failures and anomalous conditions followed by direct or indirect execution of the appropriate remediation actions.

A more technical IoT architecture

Let’s explore the different elements that shape this more technical architecture.

IoT applications

These are the actual use cases and services that benefit from IoT technology. They span a wide range of industries, including smart cities, health monitoring, industrial automation, transportation, and more. These applications rely on IoT sensors and gateways to collect and process data to offer services like predictive maintenance, asset tracking, or smart energy management. Examples of IoT applications: look at slides.

Sensors

- Sensors can measure some quantity in the environment (e.g., a thermometer).

- Actuators can do something in the environment (e.g., turn on the heating system).

- Some sensors contain sufficient functionality to perform some local operations.

- Some sensors and things are designed to connect directly to the Internet and communicate with applications and services residing in the cloud.

  - It may require external computation platforms given energy constraints.

  - Security cameras, fire sensors, thermostats, appliances, and power meters, …

Gateways

Sensors connect to the Internet using intermediaries, such as gateways. Gateways (and fog nodes) are usually more powerful devices  connection via local network links, often wireless, such as: ZigBee, variants of 802.5.14 networks, Bluetooth, and low-power Wi-Fi, LoRa, NB-IoT.

Gateways usually provide wide-area connectivity and edge processing for the attached sensors that may come in the form of protocol conversion, data storage and filtering, event processing, and analytics.

Communication layers

Enables a vast array of edge devices and things to exchange messages with each other, the rest of the IoT system, and ultimately the Internet.

Communication layers may include a variety of wireless and wired links, spanning local areas and including long-haul connections. Represent a complex infrastructure of links, bridges, and routers that can transport payloads from local point-to-point segments all the way to any endpoint and application on the Internet.

The cloud

“Back-end” processing is depicted by a generic cloud.

- Data from a variety of diverse sources are aggregated and processed for optimization and discovery of global trends and relations.

- Depending on nature and real-time requirements, sensor data may be processed “inflight” as streams, stored for post-processing and archival purposes, or both.

- May also contain some common services such as large-scale storage, analytics processing engines, data visualization and graphing, as well as management functions such as security and provisioning.

- Machine learning (ML) and artificial intelligence (AI) algorithms are usually operated in the cloud where they can work with large aggregations of data.

Control plane

- Data plane / user plane: main IoT functionalities: collect, process, and act on data.

- Control plane: task of keeping the IoT infrastructure itself running and secure.

  - Service configuration and aggregation

  - Service problem management

  - Service quality management

  - Resource provisioning

  - Trouble and anomaly management

  - Performance management

IoT: why today? A confluence of technological and infrastructure developments centered around the Internet forms much of the basis and impetus for the construction of IoT systems.

- Industry 4.0 and digitalization.

- Sensors – installed base, variety, lower cost, and easier integration.

- Multi-sensors miniaturization and mass production.

- Smart phones – sensors, gateways, and UI devices.

- Cloud computing – global and capacity on demand.

- AI and ML technologies – actionable insights with IoT data.

- Internet – technology, global infrastructure, and users.

- Pervasive communications: mobile devices always connected to the Internet.

The main problems in the IoT:

1) The energy problem

- Sensors might be placed in hard-to-reach locations: they need to be batterypowered and work for months or years.

- Sometimes they have solar panels or other energy harvesting methods, but we need to be careful: size and cost matter!

- Energy has a significant impact on every IoT operation: communication, computation, sensing, actuators...

- Due to the energy problem, sensors are often optimized to consume little power:

- We cannot operate on an always-on basis.

- We need to consider every aspect: any calculation or long transmission is expensive.

- For this reason, sensors often transmit very few bytes at relatively long intervals (no transmissions for hours or days in some applications).

- Limited processing capabilities (and so latency concerns).

2) The security problem

- The simplicity of IoT devices also makes them hard to secure.

- Impossible to run strong cryptography.

- Difficult to patch and update to solve vulnerabilities.

- Connected by design to the open Internet.

- There are possible security solutions against hackers, but there is also a privacy issue: what do sensors say about our lives? Who gets to see the data?

- Sensor data can tell a lot about our lives:

- Location data can identify movements and activities.

- Domotic systems tell stories about our habits.

- Data are often used by IoT companies and law enforcement agencies.

3) The scalability problem

- Cellular networks and wireless technologies were designed for few devices with high requirements (e.g., humans streaming videos).

- In the IoT, we have thousands or millions of devices with very little data to send.

  - New access mechanisms.

  - Sensor identification problems.

  - How do we get the data with the minimum amount of transmissions?

4) The reliability problem

- The “converse” problem (what do we need to ensure that we get the relevant information) can be crucial in industrial and safety systems:

  - How do we identify anomalous (and potentially hazardous) situations?

  - Anomalies can be single-sensor or multi-sensor.

  - How do we trust that there is an anomaly when sensors are unreliable?

## 1.2 System architecture

### 1.2.1 Structure and definitions

- Client/server: the information sources explicitly know the destination about the destination of their messages.

- Publish/subscribe: the sources send (publish) their messages on a logical distribution channel, to which zero, one or more interested destinations may subscribe, implementing a one-to-many communication pattern.

- It is not necessary to create explicit communication sessions.

- Looking at the information carried by the network, we can identify 3 flows:

  - Streams of asynchronous events (generated autonomously).

  - Measurements transmitter periodically or on-demand.

  - Commands coming from the processing platform.

Properties of a generic IoT system:

- Trustworthiness: availability, resilience, confidentiality, integrity, protection of personal information, safety.

- Architectural: composability, functional and management capability separation, heterogeneity, distribution, legacy support, modularity, network connectivity, scalability, shareability, and unique identification.

- Functional: accuracy, auto-configuration, compliance, content-awareness, context awareness, "big data" management, discoverability, flexibility, manageability, network communication, network management and operation, real-time capability, self-description, and service subscription.

Six functional domains are created:

- Physical Entity Domain (PED): physically monitored and controlled objects.

- Sensing & Controlling Domain (SCD): sensors, actuators, and adaptation devices bridging the physical and the cyber world.

- Operations & Management Domain (OMD): Operation Support Systems (OSS) + Business Support Systems (BSS); it covers the functions for provisioning, monitoring, assurance, and optimization of the operational performance of the system.

- Resource Access & Interchange Domain (RAID): interfaces through which the services are offered, under suitable access policies, to external entities.

- Application & Service Domain (ASD): services to the end-users interacting with applications, sensors, actuators and externally via the RAID. Use cloud platforms through a portal or Application Programming Interfaces (APIs).

- User Domain (UD): human and digital users, accessing services via general purpose (PCs, smartphones) or specialized (control panels, smart glasses) end-user devices.

Network architectures

Three network architectures:

- One-level architecture: the endpoints know about the addressing scheme.

  - Direct IoT connectivity to the Internet protocol stack.

  - Typically implemented with cellular network technology.

- Two-level architecture: the last segment is unaware of addressing aspects (endpoints send messages which are not geographically routable) and a gateway handles the communication protocol.

  - End nodes are generally resource and/or energy constrained (e.g., sensors).

  - Typically implemented with Low Power Wide Area (LPWA) technologies.

  - Example: LoRa, SigFox, NB-IoT, etc.

- Two-level architecture: the endpoints are not resource-constrained and implement the TCP/IP protocol stack. The traffic is managed by an intermediate router.

  - Typically implemented with Ethernet LANs or Wi-Fi, etc.

Wide area networks (WANs)

- WANs are wired TCP/IP networks.

- The WAN must be shared among many users and applications (economic reasons). Maintain integrity and data protection: mechanisms to segregate and reserve the traffic.

- Virtual Private Network (VPN): network that takes resources (bandwidth capacity, switching and routing functions, firewalls and other security appliances etc.) and reserves them to a defined group of users/endpoints. Users external to this group cannot communicate directly with the users of the group.

- Users outside of a group cannot communicate directly with users of the group.

- Use private IP addressing and private routing protocol instances.

TOPOLOGIES

1) BUS

- Every nodes taps into a common medium, signals may collide with each other.

- Need to arbitrate who will get the bus.

- The common medium is the bottleneck (to be shared) à single point of failure.

- Use CSMA/CD

- Example: (old, 10BASE2, 10BASE5) Ethernet.

2) (TOKEN) RING

- Nodes are in a ring: receive from predecessor and send to successor.

- Need to arbitrate which node can access the ring (repeaters at every link).

- The common medium is the bottleneck (to be shared) à single point of failure.

- Token-ring topology:

  - A node wishing to transmit waits for the receipt of a token, removes it from the ring, and places its message.

  - Then, the node holding the token passes it along.

  - Eliminates collisions and can increase throughput.

  - Need a mechanism to preserve token integrity and to regenerate it if necessary (e.g., when nodes are powered off).

3) STAR

- The master is at the center, other nodes are slaves linked to the master.

- Slaves communicate via master.

  - Easy to arbitrate among slaves (master decides)

  - Not scalable (the master is the bottleneck)

  - Normally for small networks or that requires predictable performance

  - Master failure shutdowns  single point of failure.

4) (PARTIAL) MESH

- Nodes are arranged according to connectivity graph.

  - Each node has a dedicated point-to-point connection to every other node.

  - Number of physical links/connections is n(n-1)/2 (if full-duplex).

  - Reduces traffic forwarding issues (+ no shared medium collisions).

  - Ensures robust and private/secure network.

- Partial mesh: a subset of links representing direct connectivity between a subset of nodes.

ENDPOINTS

A sensor is a device that detects some measurable aspect of the physical world state (stimulus) and converts it to a processable output, e.g., an electrical signal. There are thousands of sensor types, that can measure almost anything:

- Pressure, humidity, air quality

- Fingerprints, facial features, iris patterns

- Microphones, touch-sensitive surfaces, radars

- Light, smoke, chemical sensors

- Temperature

- Force

- Positioning

- …

Types of sensors:

- Fixed: stable position wrt the network topology (wireless vs. wired).

  - No need to re-authenticate, stable address and routing, simple management, etc.

  - Can be mains powered.

  - Can have permanement bad wireless connectivity conditions, if bad deployment.

- Mobile: must be connected to a cellular or LPWA network.

  - May experience a degraded service only occasionally.

  - Limited mobility constraints (e.g., handover).

  - Cannot be mains powered.

- Nomadic: can change their physical position but stay in the same place for the whole duration of a communication session.

  - Must re-authenticate, but do not require tracking functions.

- Active: need to run a current or send a signal to get a measurement (e.g., clocks, strain gauges, radar).

- Passive: use the physical phenomenon itself (e.g., piezoaccelerometers, GPS, thermocouples). This does not mean that passive sensors consume zero energy!

Actuators are an output part of the IoT system that performs direct actions. Actuation provides the means to implement control actions as and when determined by the algorithms or system operators.

- Digital actuators: On-off action. Past the appropriate signal conditioning can trigger relays or change the state of a thing, such as a power switch which can result in turning a light on or off.

- Analog actuators: Produce continuous signals that can be used to drive devices. For example, controlling the speed of a motor or producing sound in headphones.

Gateways

- It links sensors and things at the edge with higher levels of system processing hierarchy and the cloud.

- It performs or assists data collection.

- It provides Internet connectivity to transfer of sensor data to other components.

- Security boundary between things with varying levels of security.

- Optional: data storage, event and alert processing, and control (automation).

- Advanced: analytics.

Fog nodes

- Etimology: bring the cloud to the ground.

- Continuum between the edge and the cloud by placing additional computation and storage capability closer to the edge.

- Perform local processing (e.g., data reduction, filtering, and front-end analytics) with potentially lower latency than in the cloud.

- Fog nodes tend to be more powerful but are otherwise not architecturally or functionally different from the edge gateways.

Slide 22/83

### 1.2.2 Data plane functions

…

# 2. Smart Cities

…